Design of PWM Generator in Verilog

| Roll No | Name of the team Member | Sign |
| --- | --- | --- |
| 04 | Arya Mahesh Pal |  |

Date of Submission: 5-04-2026

| Abstract | The PWM (Pulse Width Modulation) Generator is designed using Verilog HDL. The design follows a behavioral modeling approach where a counter is compared with a duty cycle input to generate PWM output. Simulation is performed using Xilinx Vivado to verify correct operation for different duty cycles. |
| --- | --- |
| Introduction | ### Problem Statement :Design a PWM generator that produces a digital output signal with variable duty cycle. ### System Specifications:  8-bit duty cycle input  Output: PWM signal  Clock driven system ###  Duty cycle range: 0–100% ### Tools Used: Xilinx Vivado |
| Design Methodology | Block Diagram: +-------------+ Clk -- ¦ ¦ ¦ Counter ¦ ¦ (8-bit) ¦ +-------------+ ¦ ¦ counter value v +-----------------+ Duty ¦ Comparator ¦ ------> Pwm_out (8-bit) ¦ ( < check ) ¦ +-----------------+ Finite State Machine (FSM): FSM is not used in this design as it is based on simple sequential logic and does not involve state transitions. Module Hierarchy: The design consists of a single top-level module pwm_generator, which internally includes counter and comparator logic. |
| Verilog Implementation | Code: module pwm_generator( input clk, input [7:0] duty, output reg pwm_out ); reg [7:0] counter = 0; always @(posedge clk) begin counter <= counter + 1; if(counter < duty) pwm_out <= 1; else pwm_out <= 0; end endmodule Design Style:  Behavioral modeling used  Simple comparator logic  Efficient and synthesizable |
| Verification & Simulation Waveform | Testbench: module tb_pwm; reg clk; reg [7:0] duty; wire pwm_out; pwm_generator uut(.clk(clk),.duty(duty),.pwm_out(pwm_out)); always #5 clk=~clk; initial begin clk=0; duty=64; #100; duty=128; #100; duty=192; #100; $stop; end endmodule |
| Synthesis Results Resource Utilization Power | RTL Schematic: |
| Conclusion Future Scope | The PWM Generator was successfully designed and verified. The output waveform correctly changes with different duty cycle inputs.  Increase resolution (more bits)  Add variable frequency control  Apply in motor control / LED dimming  Optimize for low power |

| Name of Teacher | Mrs.M.N.Kalbande | Sign |  |
| --- | --- | --- | --- |
| Marks |  |  |  |